OK.. the pegs. There are two ways to get the desired results. Buy the peg-lowering kit which is easy yet expensive ($130+) or you can remove the rear and put them up front which is a little cheaper. Total cost this way.. (2) new rear pegs $55+ shift lever extension $25. Tot.$80+ shipping (From Bob’s BMW)

If you decide to do this one thing you have to have is on-site access to a power grinding wheel. The procedure goes like this.

1.Remove the front pegs. Set aside. You will need the pins, clips, and the rubber top section to put on your new rear pegs that you are gonna have to order. The rubber is interchangeable.

2. Remove the rear pegs. Be careful not to let the holding clips fly off, but more importantly, be very careful not to lose the little SPRINGS and BALLS found inside and underneath the rear pegs. You will need them when you replace the pegs.

3. Once you get the rear pegs off and try them up front... you’ll be able to see that in order for you to install the holding pin... you are gonna have to do some grinding. This is really a trial and error procedure as to how much has to be ground off. I took it VERY slow. It is imperative that you grind a FLAT surface in order to get a good fit to the bike. Meaning: Try not to round off the edges with the grinder.  Grind and fit.... grind and fit.. Etc... You’ll see what I mean when you get into it. Once you get the hole to line up and the pin goes in.. You’ll feel like Bob Vila!  Really, it’s not a difficult job. The second peg comes along quicker than the first. Took me a little under 2 hours.

The only downside of lowering your pegs this way is that your pegs will not be spring loaded like the stocker. No big deal to me. “IF” you decide to do this... go ahead and call Bob’s BMW (800-BMW-BOBS) and order the shift relocation kit (part# WRK12SRK). IMHO..You will have to have it. Some fellow owners said they didn’t need it, but I couldn’t even shift without it. I have no idea what kind of ergonomics they’re used to or what kind of feet they possess, but to me, this was a necessity. Geez.

Good luck my friend.